Embedded microchannel arrays for contrast agent transport in silicone MRI phantoms

###### D. Williams1, D. Yang1, B. Leverton2, R. Little2, B. Dickie3,4, and B. Derby1,*

1 Department of Materials, School of Natural Sciences, Faculty of Science and Engineering and Henry Royce Institute, 
  The University of Manchester, UK

2 Clinical & Pre-Clinical Imaging, Faculty of Biology, Medicine and Health, The University of Manchester, 
  Manchester, UK

3 Division of Pharmacy & Optometry, School of Health Sciences, The University of Manchester, Manchester, UK

4 Division of Informatics, Imaging & Data Sciences, School of Health Sciences, 
  The University of Manchester, Manchester, UK

* Corresponding author, email: brian.derby@manchester.ac.uk

© 2026 Brian Derby; licensee Infinite Science Publishing

This is an Open Access abstract distributed under the terms of the Creative Commons Attribution License, which permits unrestricted use, distribution, and reproduction in any medium, provided the original work is properly cited (http://creativecommons.org/licenses/by/4.0).

Clinically relevant MRI phantoms play a key role in the development of imaging techniques and surgical practice, using tissue mimicking materials to replicate the architectures and signal response for a specific anatomical region [1], [2]. Tailoring the physical and chemical properties of these materials to match the magnetic resonance relaxation times, T1 and T2, of native tissues is fundamental for the development of pathophysiological assessments (e.g. dementia), and monitoring the progression and treatment of disease (e.g. cancer and arthritis) [3], [4]. The detection of tumours with MRI relies on the accumulation of contrast agents in the extravascular and extracellular space, producing higher signal intensities than the surrounding healthy tissue. This plays a pivotal role in evaluating pharmokinetic parameters in patients [4], however, is not yet replicated in MRI phantoms. To realistically reproduce anatomical size-scales, sub-voxel features are required within macro-scale phantoms [5] such as micro-channels to form perfusion phantoms [2], [6]. In this work, we used replica lithography and sacrificial wax moulding to embed microchannel arrays (D: 500-1000 μm) into a silicone phantom to simulate the perfusion of a gadolinium contrast agent, imaged using a 7 Tesla pre-clinical MRI scanner. We attempted to simulate interstitial tissue accumulation and characterise the signal enhancement of an adjacent poly(ethylene glycol) diacrylate phase in response to contrast agent permeation. We mimicked the arterial bolus and demonstrated contrast agent accumulation, with full- and partial-volume voxel (1x1x1 mm) assessment reflecting the spatial movement of gadolinium. Fabricating sub-voxel features remains technically challenging, however, combined with permeable secondary phases, they may replicate the role of capillary networks in contrast agent exchange within tumours.

### Author’s statement

Conflict of interest: The authors state no conflict of interest. Animal models: No in-vivo experiments were involved with this work. Informed consent: No human participants were involved with this work. Ethical approval: No human/in-vivo participants were involved with this work. Acknowledgments: We acknowledge the Pre-Clinical Imaging Facility at the University of Manchester for providing access to their 7T MRI system (Bruker Avance III console / 16-cm horizontal bore Magnex magnet), which was instrumental in the completion of this research. Research funding: This research was funded by the RoseTree Foundation, ID2021\100049.

References

[1] H. Yusuff et al., Narrative review of tissue-mimicking materials for MRI phantoms: Composition, fabrication, and relaxation properties, Radiography, vol. 30, no. 6, p. 1655–1668, 2024.

[2] 	J. Coles-Black et al., Accessing 3D Printed Vascular Phantoms for Procedural Simulation, Front. Surg, vol. 7, p. 626212, 2021.

[3] 	K. Stupic et al., A standard system phantom for magnetic resonance imaging, Magn. Reson. Med, vol. 86, no. 3, p. 1194–1211, 2021.

[4] 	M. Sarwar et al., 3D-printable phantoms for quantitative dynamic contrast‐enhanced MRI, Magn. Reson. Med, vol. 94, no. 5, p. 1889–1899, 2025.

[5] 	C. Sainz Martinez et al., BigBrain MR: a new digital phantom with anatomically-realistic magnetic resonance properties at 100-µm resolution for magnetic resonance methods development, NeuroImage, vol. 273, p. 120074, 2023.

[6] 	S. Soloukey et al., Patient-Specific Vascular Flow Phantom for MRI- and Doppler Ultrasound Imaging, Ultrasound Med. Biol, vol. 50, no. 6, p. 860–868, 2024.